PSActiveX Function Context

PSActiveX is designed for the second exploitation and usage to PowerMS, control has achieved the all functions which PowerMS software has, the mainly usages please refer to the following instruction.

PSActiveX Soft Context

- Register OCX Commponet

regsvr32  PSActiveXServer.ocx

- Install OCX for Delphi:

Select “Component | Import ActiveX Control” item, select“PSServerProj1 Library” and install.

- Install OCX for VC:

Select “Project | Add to Project | Components and Controls” item，Select “Registered ActiveX

and Controls”, Select “PSServer Control file name” and Insert ！

4． Install OCX for VB

Select “Project | Components” item, Select “Controls” Page and Select “PSServerProj Library”

- Primary method:

1) Open COM:

PS_OpenComm(Port, BaudRate, ByteSize, Parity, StopBits: Integer);

Port:COM口

BaudRate：Communication Speed(4800，9600，19200，38400)

ByteSize： Data Length（Default：8）

Parity：Parity bit（Default 0 ：No Parity）

StopBits：Stop bit（Default 0：One Stop bit）

2）Close COM

PS_CloseComm;

3）Read Power Params

PS_ReadParams;

4）Setting Power Params

PS_SetParams(CurMax, VolMax, PowMax, VolSetting,PowAdd: Integer);

CurMax：Current MAX（0-3000）MA

VolMax： Voltage MAX（0-36000）MV

PowMax：Power MAX（0-10800 -> 0-108.00W）

PowAdd：Power New Address

5）Control Power State：

PS_ControlState(State: Integer);

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

0bit：Power Output State，0->On，1->Off

1bit： PC Control State，0->Self，1->PC

6）Get Execute Result

PS_GetExecResult: SYSINT;

Return Value  ：1->OK，0->Failed

7）Setting Power Address（0-255）

PS_SetDeviceAdd(Address: Integer);

8）Get Power Params（VC & Delphi）

PS_GetData(out CurValue, CurMax, VolValue, VolSetting, VolMax,PowValue, PowMax, State: Integer);

CurValue：Currently Current Value(0-3000mA)

VolValue：Currently Voltage Value(0-36000mV)

VolSetting：Setting Voltage Valye（0-36000mV）

VolMax：Setting Voltage MAX Value（0-36000mV）

PowValue：Power Settings MAX Value（0-10800  => 0- 108.00W

State：Power State

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

0bit：Output State，0->On，1->Off

1bit：Power Current Alarm State，0->Normal，1->exceptional

2bit：Power Power Alarm State，0->Normal，1->exceptional

3bit：Control State，0->Self，1->PC Control

Get Params (VB)

VolValue   ： PS_Get_VolCurrent

VolMax   ： PS_Get_VolMax

VolSetting   ： PS_Get_VolSetting

CurValue   ： PS_Get_CurCurrent

CurMax   ： PS_Get_CurMaxt

PowValue   ： PS_Get_PowCurrent

PowMax   ： PS_Get_PowMax

State   ： PS_Get_State

Attention: If you don’t use ActiveX control to program, you should set the DTR（Pin4）and RTS(Pin7) of COM to high voltage, or you can’t communicate successfully.